Lab 6: Induction Machines

Aman Webeshet and James Nicholls

Objective

In this lab, we will be examining the different characteristics of the induction motor and generator at different levels of loading and terminal voltages. We will also determine the

relationship between input and output power for a given speed and torque conditions.

Motor Name Plate and Predicted Values

Full Load Power = 746W * 2.5HP = 1.865 KW

Torque=PoutWm=745.7*2.52*pi*169060*1lbs-ft1.35582N-m=7.77lb-ft

Nsync= 120*604 = 1800 rev/min

Slip = 1800-16901800*100%=6.11%

| Estimated parameters (per phase equivalent circuit) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| R1 | X1 | Rc | Xm | X2 | R2 |
| 1.62 | 1.202 | 0.4132 | 30.306 | 1.202 | 0.130 |

R1+R2 = PI2= 106.3W7.792A=1.75 ohm

X1+X2 = VavgI12- R1+R22= 23.27.792- 1.752=j2.403 ohm

Zeq = QI12= 449.4VAR3.792 A=31.508 ohm

R1 = 1.62 (found from Part A)

X1 = X2 =  X1+X22= 2.403j2 = 1.202j

Rc = PavgI12-R1= 293.782-1.75 = 0.413 ohm

Xm = Zeq – X1 = 31.508-1.202 = 30.306j

R2 = (R1 +R2) – R1 = 1.75 – 1.62 = 0.13 ohm

Part 1: Stator Resistance Measurement

Figure 1: Circuit diagram for induction machine stator resistance measurement

R1 = 1.62 ohm

Part B: Locked Rotor Test (Short Circuit Test)

R1+R2 = PI2= 106.3W7.792A=1.75 ohm

X1+X2 = VavgI12- R1+R22= 23.27.792- 1.752=j2.403 ohm

Part C: Operation Characteristics at varying loads

SAME WIRING AS PART B

Part D: Synchronous Operation Test (Rotor Open Circuit Test)

Zeq = QI12= 449.4VAR3.792 A=31.508 ohm

Rc = PavgI12-R1= 293.782-1.75 = 0.413 ohm

Part E: Induction Generator

SAME WIRING AS PART D

We get negative Torque because the force is in the opposite direction as the motor is now a generator in this setup

Questions